Cornerstone Electronics Technology and Robotics I Week 14
Combination Circuits Tutorial (Series-Parallel)
· Administration:

· Prayer

· Turn in quiz

· Electricity and Electronics, Chapter 8, Introduction:
· In this lesson, we will learn to analyze circuits with resistors in both series and parallel together and compute the total resistance         (equivalent resistance) for those circuits.  From there, we can calculate the voltage drop and current for each resistor in the circuit.  
· Series-parallel circuits are also called combination circuits.

· Examples:

[image: image1.jpg]



[image: image2.jpg]R2

R3

AW

AW

AW

R4






    Figure 1




Figure 2

· See applets:
·  http://www.falstad.com/circuit/e-resistors.html 
· http://webphysics.davidson.edu/physlet_resources/bu_semester2/c10_combo.html
· Regardless of the complexity of a series-parallel circuit, it can be solved when broken down into steps of individual series and parallel circuits. 
· Review:

· Four equations are used to solve series resistor circuits.  They are:





RT = R1 + R2 + R3 + … + RN




VT = V1 + V2 + V3 + …+ VN




IT = I1 = I2 = I3 =…IN




V = I x R




V = I x R can be applied to the total circuit (VT = IT x RT) and to 


individual resistors (V1 = I1 x R1).



See Lesson 12, Series Circuits for details.  Link:  http://www.cornerstonerobotics.org/curriculum/lessons_year1/ER%20Week12,%20Series%20Circuits.pdf
· Four equations are used to solve parallel resistor circuits.  They are:

1/RT = 1/R1 + 1/R2 + 1/R3 +………..+1/RN





VT = V1 = V2 = V3 = ….. VN




IT = I1 + I2 + I3 + …+ IN




V = I x R



See Lesson 13, Parallel Circuits for details.  Link:  http://www.cornerstonerobotics.org/curriculum/lessons_year1/ER%20Week13,%20Parallel%20Circuits.pdf
· Power in Combination Circuits: The total power in a combination circuit is equal to the sum of the individual powers.

· The variables used in Ohm's Law equations must be common to the same two points in the circuit under consideration.  This rule cannot be overemphasized. This is especially important in series-parallel combination circuits where nearby components may have different values for both voltage drop and current.

· Electricity and Electronics, Section 8.1, Solving a Complex Circuit:
· The goal of series-parallel resistor circuit analysis is to be able to determine all voltage drops, currents, and power dissipations in a circuit.
· Contracting the Circuit: Finding the total resistance and total current:  In this section, you will combine series and parallel resistors in a step by step process until the entire circuit is reduced to one equivalent resistor.
· Step 1:   Assess which resistors in a circuit are connected together in simple series or simple parallel.

· Step 2:  Re-draw the circuit, replacing each of those series or parallel resistor combinations identified in step 1 with a single, equivalent-value resistor.

· Step 3:  Repeat Steps 1 and 2 until the entire circuit is reduced to one equivalent resistor.

· Step 4:  Find the total current for the circuit.

· Expanding the Circuit:  In this section, you will proceed step-by-step from the simplified version of the circuit back into its original form, plugging in values of voltage and current where appropriate until all values of voltage and current are known.
· Step 5:  Taking total voltage and total current values, go back to last step in the circuit reduction process and insert those values where applicable. 
· Step 6:  From known resistances and total voltage / total current values from step 5, use Ohm's Law to calculate unknown values (voltage or current)
· Step 7:  Repeat steps 5 and 6 until all values for voltage and current are known in the original circuit configuration. Essentially, you will proceed step-by-step from the simplified version of the circuit back into its original, complex form, plugging in values of voltage and current where appropriate until all values of voltage and current are known.

· Step 8:  Calculate power dissipations from known voltage, current, and/or resistance values.
· Example 1:  Parallel resistors R1 and R2 are in series with resistor R3. See Figure 8.1.


 (Note:  In the following examples, when you encounter a 


resistor labeled R1,2, it is an equivalent resistor for resistors R1 

and R2. R1,2,3,4,5 is the equivalent resistor for resistors R1, R2, R3, 

R4, and R5.  Similarly, V4,5 is the voltage drop across the 


equivalent resistor R4,5.)
[image: image3.jpg]BAT1

MW
100

R2

MW
100

R3

50




Figure 8.1


Solve for the all of the unknowns in Table 8.1.

[image: image4.jpg]Resistance | Voltage | Current
R, 100 Q V, [}
R, 100 Q V, 1,
Rs 50 Q Vs ls
Total Ry 50V I





Table 8.1
· Contracting the Circuit: Finding the Total Resistance, RT  and Total Current, IT :
· Step 1:   Assess which resistors in a circuit are connected together in simple series or simple parallel.  In our circuit, R1 and R2 are in a simple parallel configuration.
· Step 2:  Re-draw the circuit, (Figure 8.2), replacing each of those series or parallel resistor combinations identified in Step 1 with a single, equivalent-value resistor.  R1 and R2 are replaced with R1,2.
[image: image5.jpg]R1.2

50





Figure 8.2
R1,2 = R/N

R1,2 = 100 /2

R1,2 = 50 
· Step 3:  Repeat Steps 1 and 2 until the entire circuit is reduced to one equivalent resistor.  (Step 1) - R1,2 and R3 are in a simple series configuration.  (Step 2) - Re-draw the circuit, (Figure 8.3), with R1,2,3 replacing R1,2 and R3.  Enter RT, (R1,2,3), into the solution table (Table 8.2).
[image: image6.jpg]004





Figure 8.3



RT = R1,2,3 



RT = R1,2 + R3





RT = 50 + 50 





RT = 100 

[image: image7.jpg]Resistance | Voltage | Current
R1 [ R1=100Q V1 11
R2 | R2=100Q V2 12
R3 | R3=500Q V3 13
Total| Rr=100Q | V=50V It





Table 8.2     
· Step 4:  Find the total current for the circuit.





IT = VT / RT





IT = 50 V / 100 





IT = 0.5 A



See Table 8.3:

[image: image8.jpg]Resistance | Voltage | Current
R1 [ R1=100Q V1 11
R2 | R2=100Q V2 12
R3 | R3=500Q V3 13
Total| Rr=100Q | V=50V | Ir=0.5A





Table 8.3
· Expanding the Circuit:  Find all remaining values of voltage and current. 
· Step 5:  Taking total voltage and total current values, go back to last step in the circuit reduction process, (Figure 8.4), and insert those values where applicable.  We can now determine the current through each series resistor in the equivalent circuit in Figure 8.4.

[image: image9.jpg]R1.2

50





Figure 8.4






IT = I1,2 = I3 = 0.5 A



Enter the value of I3 into the solution table (Table 8.4).

[image: image10.jpg]Resistance | Voltage | Current
R1 [ R1=100Q V1 11
R2 | R2=100Q V2 12
R3 | R3=500Q V3 13=05A
Total| Rr=100Q | V=50V | IT=05A





Table 8.4
· Step 6:  From known resistances and total voltage / total current values from Step 5, use Ohm’s Law to calculate unknown values (voltage or current).  In our example, calculate the voltage drop V3 across the series resistor R3 and the voltage drop V1,2 across the parallel combination (equivalent resistor, R1,2).  
[image: image11.jpg]R1.2

50





Figure 8.5





V3 = I3 x R3










V3 = 0.5 A x 50 





V3 = 25 V






V1,2 = I1,2 x R1,2





V1,2 = 0.5 A x 50 





V1,2 = 25 V



Since R1 and R2 are parallel resistors, the voltage drops V1 and 


V2 are the same value and equal to V1,2.

V1 = V2 = V1,2 = 25 V


We can now enter V1, V2, and V3 into the solution table (Table 8.5).
[image: image12.jpg]Resistance | Voltage | Current
R1 [R1=100Q [V1=25V 11
R2 | R2=100Q [V2=25V 12
R3 | R3=50Q |V3=25V|[I3=05A
Total| Rr=100Q | V=50V | IT=05A





Table 8.5
· Step 7:  Repeat steps 5 and 6 until all values for voltage and current are known in the original circuit configuration. Essentially, you will proceed step-by-step from the simplified version of the circuit back into its original, complex form, plugging in values of voltage and current where appropriate until all values of voltage and current are known.  Here, calculate the current through each parallel resistor (R1 and R2) in the parallel resistor combination.  Use Ohm’s Law.
[image: image13.jpg]BAT1

MW
100

R2

MW
100

R3

50




Figure 8.6





I1 = V1 / R1







I1 = 25 V / 100 





I1 = 0.25 A






I2 = V2 / R2







I2 = 25 V / 100 





I2 = 0.25 A



Enter I1 and I2 into the solution table (Table 8.6).

[image: image14.jpg]Resistance | Voltage Current
R1 [R1=100Q | V1=25V [11=0.25 A
R2 [R2=100Q | V2=25V [12=0.25 A
R3 | R3=500Q | V3=25V | I3=05A
Total| Rr=100Q | V=50V | Ir=05A





Table 8.6
· Step 8:  Calculate power dissipations from known voltage, current, and/or resistance values.





P1 = V1 x I1





P1 = 25 V x 0.25 A





P1 = 6.25 W






P2 = V2 x I2





P2 = 25 V x 0.25 A






P2 = 6.25 W






P3 = V3 x I3





P3 = 25 V x 0.5 A






P3 = 12.5 W






PT = VT x IT





PT = 50 V x 0.5 A






PT = 25 W




The final solution table (Table 8.7):

[image: image15.jpg]Resistance | Voltage Current Power
R1 |R1=100Q [V1=25V|[I1=025A|P1=6.25W
R2 | R2=100Q |V2=25V |[I12=025A|P2=6.25W
R3 | R3=50Q [V3=25V | I13=05A |P3=125W
Total| Rr=100Q | Vr=50V | IT=05A | Pr=25W





Table 8.7


Summary of Values:

[image: image16.jpg]Vr =50V
IT=05A z
Rr=100Q ©
Pr=25 W





Figure 8.7
· Example 2:  This combination circuit is a more complicated set of series and parallel resistors.  See Figure 8.8.
[image: image17.jpg]100V

BAT1

RE

MWW
100
R2 R3
MWW
100 50
R4
56
RS

56

150




Figure 8.8



Solve for the all of the unknowns in Table 8.8.

[image: image18.jpg]Resistor | Resistance | Voltage | Current
R1 R1=100Q \'Al 1
R2 R2 =100 Q \'7] 12
R3 R3=500 V3 13
R4 R4 =56 Q V4 14
R5 R5=56 Q V5 15
R6 R6 = 150 Q V6 16

Totals Rr V=100V Ir





Table 8.8

· Contracting the Circuit: Finding the Total Resistance, RT  and Total Current, IT :
· Step 1:   Assess which resistors in a circuit are connected together in simple series or simple parallel.  In this circuit, R2 and R3 are in simple series in a parallel branch.  Also, R4 and R5 are in simple parallel configuration.


· Step 2:  Re-draw the circuit, (Figure 8.9), replacing each of those series or parallel resistor combinations identified in Step 1 with a single, equivalent-value resistor. 
[image: image19.jpg]051

=]

[A%]

A00OL





Figure 8.9



To determine the equivalent resistance R2,3, add the series 



resistors R2 and R3 together.





R2,3:





R2,3 = R2 + R3





R2,3 = 100  + 50 





R2,3 = 150 



Also determine the equivalent resistor R4,5 for the 




parallel resistors, R4 and R5, using the formula for parallel 



resistors of equal value.





R4,5:

R4,5 = R/N





R4,5 = 56 / 2






R4,5 = 28 
· Step 3:  Repeat Steps 1 and 2 until the entire circuit is reduced to one equivalent resistor.  (Step 1) - R1 and R2,3 are in a simple parallel configuration.  R4,5 and R6 are in a simple series configuration. (Step 2) - Re-draw the circuit, (Figure 8.10), with R1,2,3 replacing R1 and R2,3 and R4,5,6 replacing R4,5 and R6. [image: image20.jpg]o
o;

&

178




               Figure 8.10



Determine the equivalent resistor R1,2,3 for the parallel 



resistors, R1 and R2,3, using the formula for two parallel 



resistors.  





R1,2,3:
R1,2,3 = R1 x R2,3 / R1 + R2,3





R1,2,3 = 100  x 150  / 100  + 150 





R1,2,3 = 15,000 2 / 250 








R1,2,3 = 60 




Determine the equivalent resistor R4,5,6 for the series resistors, 


R4,5 and R6, using the formula series resistor formula.





R4,5,6:





R4,5,6 = R4,5 + R6





R4,5,6 = 28 + 150 





R4,5,6 = 178 

· Step 3 again:  Repeat Steps 1 and 2 until the entire circuit is reduced to one equivalent resistor.  (Step 1) - R1,2,3 and R4,5,6 are in a simple series configuration. (Step 2) - Re-draw the circuit, (Figure 8.11), with R1,2,3,4,5,6 replacing R1,2,3 and R4.5.6.  
[image: image21.jpg]100V

R1,23,456

238





Figure 8.11



Determine the equivalent resistor R1,2,3,4,5,6 for the series 



resistors R1,2,3 and R4,5,6 using the formula series resistor 



formula.






R1,2,3,4,5,6 = R1,2,3 + R4,5,6





R1,2,3,4,5,6 = 60 + 178





R1,2,3,4,5,6 = 238 






R1,2,3,4,5,6 = RT



Enter RT, (R1,2,3,4,5,6), into the solution table (Table 8.9).
[image: image22.jpg]Resistor | Resistance | Voltage | Current
R1 R1=100Q V1 11
R2 R2 =100 Q V2 12
R3 R3=500 V3 13
R4 R4 =56 Q V4 14
R5 R5=56 Q V5 15
R6 R6 =150 Q V6 16

Totals | Rr=238Q | Vr=100V i





Table 8.9
· Step 4:  Find the total current for the circuit.






IT = VT / RT





IT = 100 V / 238 





IT = 0.42 A




See Table 8.10:

[image: image23.jpg]Resistor | Resistance | Voltage Current
R1 R1=100Q V1 11
R2 R2 =100 Q V2 12
R3 R3=500 V3 13
R4 R4 =56 Q V4 14
R5 R5=56 Q V5 15
R6 R6 =150 Q V6 16
Totals | Rr=238Q [ Vr=100V |Ir=0.42A





Table 8.10
· Expanding the Circuit:  Find all remaining values of voltage and current. 
· Step 5:  Taking total voltage and total current values, go back to last step in the circuit reduction process, (Figure 8.12), and insert those values where applicable.  
[image: image24.jpg]o
o;

&

178




Figure 8.12




Determine the current through each series resistor in the 



equivalent circuit in Figure 8.12.






IT = I1,2,3 = I4,5,6 = 0.42 A




Also, we can calculate the voltage drop V1,2,3 across  




resistor R1,2,3.





V1,2,3 = I1,2,3 x R1,2,3





V1,2,3 = 0.42 A x 60 





V1,2,3 = 25.2 V
· Step 6:  From known resistances and total voltage / total current values from Step 5, use Ohm’s Law to calculate unknown values (voltage or current).  See Figure 8.13.

[image: image25.jpg]051

=]

[A%]

A00OL





Figure 8.13

R1 and R2,3:



Since R1 and R2,3 are parallel resistors, the voltage drops V1 



and V2,3 are the same value and equal to V1,2,3.  






V1 = V2,3 = V1,2,3 = 25.2 V




Knowing V1 and R1, solve for I1:






I1 = V1 / R1





I1 = 25.2 V / 100 





I1 = 0.25 A




The current through R2,3 is the total current IT less I1.






I2,3 = IT - I1





I2,3 = 0.42 A – 0.252 A





I2,3 = 0.168 A




I2,3 could have been solved using Ohm’s Law, I2,3 = V2,3 / R2,3


R4,5 and R6:



R4,5 and R6 are in series so they have the same current 



through them as R4,5,6.






I4,5,6 = I4,5 = I6 = 0.42 A




Knowing I6 and R6, solve for V6:






V6 = I6 x R6











V6 = 0.42 A x 150 





V6 = 63 V




Knowing I4,5 and R4,5, solve for V4,5:






V4,5 = I4,5 x R4,5











V4,5 = 0.42 A x 28 





V4,5 = 11.8 V



Update the solutions table with the values of V1, I1, V6 and I6 



(Table 8.11).
[image: image26.jpg]Resistor | Resistance | Voltage Current

R1 R1=1000Q | V1=252V |11=0.252A

R2 R2 =100 Q V2 12
R3 R3=50Q V3 13
R4 R4 =56 Q V4 14
R5 R5=56 Q V5 15

R6 R6=150Q | V6=63V | 16=0.42A

Totals | Rr=238Q | Vr=100V | IT=042A





Table 8.11
· Step 7:  Repeat steps 5 and 6 until all values for voltage and current are known in the original circuit configuration.
[image: image27.jpg]100V

BAT1

RE

MWW
100
R2 R3
MWW
100 50
R4
56
RS

56

150




Figure 8.14


R2 and R3:



R2 and R3 are in series so they have the same current 



through them as R2,3.






I2,3 = I2 = I3 = 0.168 A



Knowing I2 and R2, solve for V2:






V2 = I2 x R2











V2 = 0.168 A x 100 





V2 = 16.8 V




Knowing I3 and R3, solve for V3:






V3 = I3 x R3











V3 = 0.168 A x 50 





V3 = 8.4 V



R4 and R5:




Since R4 and R5 are parallel resistors, the voltage drops V4 



and V5 are the same value and equal to V4,5.  






V4 = V5 = V4,5 = 11.8 V




Knowing V4 and R4, solve for I4:






I4 = V4 / R4





I4 = 11.8 V / 56 





I4 = 0.21 A




Since R4 and R5 are equal, 






I4 = I5



Record voltages and currents into the solution table 




(Table 8.12).
[image: image28.jpg]Resistor | Resistance | Voltage Current
R1 R1=100Q | V1=252V |11=0.252A
R2 R2=1000Q | V2=16.8V | 12=0.168 A
R3 R3=500Q | V3=84V |I13=0.168 A
R4 R4=56Q |V4=118V | 14=0.21A
R5 R5=56Q |V6=11.8V | I15=0.21A
R6 R6=150Q | V6=63V | I6=0.42A

Totals | Rr=238Q | Vi=100V | IT=042A





Table 8.12

· Step 8:  Calculate power dissipations from known voltage, current, and/or resistance values.





P1 = V1 x I1





P1 = 25.2 V x 0.252 A






P1 = 6.3 W






P2 = V2 x I2





P2 = 16.8 V x 0.168 A






P2 = 2.8 W






P3 = V3 x I3





P3 = 8.4 V x 0.168 A






P3 = 1.4 W






P4 = V4 x I4





P4 = 11.8 V x 0.21 A






P4 = 2.5 W






P5 = V5 x I5





P5 = 11.8 V x 0.21 A






P5 = 2.5 W






P6 = V6 x I6





P6 = 63 V x 0.42 A






P6 = 26.5 W






PT = VT x IT





PT = 100 V x 0.42 A






PT = 42 W


Check power total:




PT = P1 + P2 + P3 + P4 + P5 + P6



PT = 6.3 W + 2.8 W + 1.4 W + 2.5 W + 2.5 W + 26.5 W




PT = 42.0 W



The final solution table (Table 8.13):

[image: image29.jpg]Resistor | Resistance | Voltage Current Power
R1 R1=100Q [V1=252V|I11=0.252A| P1=6.3W
R2 R2=100Q [V2=16.8V |12=0.168A | P2=2.8W
R3 R3=50Q | V3=84V |I13=0.168A| P3=14W
R4 R4=560Q [V4=118V| 4=021A | P4=25W
R5 R5=56Q |V5=118V | I5=021A | P5=25W
R6 R6=150Q | V6=63V | 1I6=042A |P6=26.5W

Totals | Rr=238Q | V=100V | IT=042A | Pr=42.0W





Table 8.13
Summary of Values:
[image: image30.jpg]Vi=252V
11=0.252A

BAT1





Figure 8.15
· Example Problem 1:

· Solve for all of the unknowns in the following circuit.  Fill in each unknown in the table below the circuit.

[image: image31.jpg]BAT1

33

R2

22

R4

47




[image: image32.jpg]Resistance

Voltage Current Power
R1 | R1=33Q
R2 | R2=15Q
R3 | R3=220Q
R4 | R4=470Q
Total V1 =24V





· Remember, to solve series circuit problems, you can use any or all of the following equations:





RT = R1 + R2 + R3 + … + RN




VT = V1 + V2 + V3 + …+ VN




IT = I1 = I2 = I3 =…IN




V = I x R
· Remember, to solve parallel circuit problems, you can use any or all of the following equations:


1/RT = 1/R1 + 1/R2 + 1/R3 +………..+1/RN





VT = V1 = V2 = V3 = ….. VN




IT = I1 + I2 + I3 + …+ IN




V = I x R
· Example Problem 2:

· Solve for all of the unknowns in the following circuit.  Fill in each unknown in the table below the circuit.

[image: image33.jpg]100

R2

100

MW
200

R6

R4

RS

a7

82




[image: image34.jpg]Resistance Voltage Current Power
R1 | R1=1000Q
R2 | R2=200Q
R3 | R3=1000Q
R4 | R4=56Q
R5 | R4=820Q
R6 | R4=47Q
Total V=24V





· Remember, to solve series circuit problems, you can use any or all of the following equations:





RT = R1 + R2 + R3 + … + RN




VT = V1 + V2 + V3 + …+ VN




IT = I1 = I2 = I3 =…IN




V = I x R
· Remember, to solve parallel circuit problems, you can use any or all of the following equations:


1/RT = 1/R1 + 1/R2 + 1/R3 +………..+1/RN





VT = V1 = V2 = V3 = ….. VN




IT = I1 + I2 + I3 + …+ IN




V = I x R
· Example Problem Solutions:

· Example Problem 1:

[image: image35.jpg]Resistance Voltage Current Power
R1 | R1=33Q | V1=1748V | 1=0530A | P1=925W
R2 | R2=15Q | V2=6.52V | 12=0435A | P2=284W
R3 | R3=22Q | V3=208V | I3=0.095A | P3=0.20W
R4 | RA=47Q | V4=444V | 14=0095A | P4=042W
Total| Rr=453Q | Vr=24V Ir=053A | Pr=1271W





· Example Problem 2:

[image: image36.jpg]Resistance Voltage Current Power
R1 R1=100Q V1=666V |11=0.0666A |P1=0443W
R2 R2=200Q | V2=13.31V |12=0.0666 A |[P2=0.886 W
R3 R3 =100 Q V3=666V | I3=0.666A |P3=0443W
R4 R4=56Q V4=443V | 14=0.079A |P4=0.350W
R5 R5=82Q V5=443V | 15=0.054 A |P5=0.239W
R6 R6=47Q V6=6.26V |16=0.1331 A |P6=0.833W
Total| Rt =180.28 Q Vr=24V It=0.1331 A [Pr=3.194 W





PAGE  
21

