Resistor Tutorial

Cornerstone Electronics Technology and Robotics I Week 8
· Administration:

· Prayer

· Turn in quiz
· Electricity and Electronics, Section 3.5, Resistors:
· Function:  Resistors restrict the flow of electric current, for example a resistor is placed in series with a light-emitting diode (LED) to limit the current passing through the LED.  They convert electrical energy into mechanical energy (heat).
· Resistors are used to:

· Provide a voltage drop

· Provide a current limit

· Dissipate (converting) electrical energy

· Some electrical components have resistance that varies with temperature or light.  They serve as sensors used in various applications.

· A larger value in ohms represents a larger resistance.
· Fixed Resistors:

· Example and Symbol:

[image: image1.jpg]



                   [image: image2.jpg]



· Carbon composite resistor

· Thin film resistor

· Film resistors

· Resistors networks:
· SIP - Single In-line Package
· DIP - Dual In-line Package
[image: image3.jpg]



       [image: image4.jpg]



          SIP



                 DIP

· Wire wound resistors are used where the resistor has to dissipate a lot of heat.
[image: image5.jpg]4-Band Resistor Color Code

First Digit Band

Second Digit Band
Multiplier Band (number of zeros)

Tolerance Band

First Digit Second Digit Multiplier Tolerance Color
# of Zeros
] [ [t
B En
“ “ n/a Grey
9 9 n/a White
x0.1 Gold
Examples: x001 19% e
AL
100 5% 2200 10% 5 6 0000 5%

Brown - Black - Brown - Gold
1 0 0 5%

100 Q +/- 5%

Red - Red - Red - Silver
2 2 00 10%

2,200 Q (2.2K) +/-10%

Green - Blue - Yellow - Gold
5 6 0000 5%

560,000 Q2 (560K) +/- 5%





[image: image6.jpg]5-Band Resistor Color Code

First Digit Band

Second Digit Band

Third Digit Band

Multiplier Band (number of zeros)
L_ Tolerance Band

First Digit Second Digit Third Digit

Examples:

Multiplier Tolerance Color
# of Zeros
o
[[4 ] [4zeros] Yellow
Ex
“ WES Grey
9 n/a White
[ xo01 | Gold
x 0.01 Silver

—ULLD-

12000 05%

Brown - Red - Black - Red - Green
1 2 0 00 05%

12,000 Q (12.0K) +/- 0.5%

4 27000 0.25%

Yellow - Red - Violet - Orange - Blue
4 2 7 000 0.25%

427,000 Q (427KK) +/-0.25%

3 35x0.101%

Orange - Orange - Green - Gold - Violet
3 3 5 x01 0.1%

335Q+-0.1%





· See resistor color code applet: 
· http://www.dannyg.com/examples/res2/resistor.htm
· Percentage and Tolerance Calculations:
· Percent means "out of 100."  For example, instead of saying "30 out of every 100 professional basketball players are female," we can say "30% of professional basketball players are female."

· 1% of anything is one hundredth part of it.
· Here are three ways to write the same thing: 

25% = 25/100 = 0.25 
To convert percent to a decimal, move the decimal to the left 2 places.
· Find 10% of 470:

· First change 10% to a decimal by moving the decimal point 2 places to the left.

10% = 0.10

· Then multiply,

0.10 x 470 = 47

· 10% of 470 is 47

· Find the upper and lower limits for a 470  resistor with a 10% tolerance:

· The allowable resistance within tolerance can be from 470  + 10% to 470  – 10%. 

· From above, 10% of 470 is 47

· Upper Limit:  470  + 10% =  470  + 47 = 517 
· Lower Limit:  470  – 10% = 470  – 47 = 423 
· Therefore the range for a 470  resistor with a 10% tolerance is from 423 to 517 ohms.
· Perform Resistors Lab 1 – Resistor Color Code      
· Resistor Failure:

· Open circuit
· Changed in value
· They never go short circuit.
· Physical characteristics:

· Different resistance values are created by mixing or doping different impurities with the carbon.

· Electrical current running through a resistor causes it to become heated.  Resistors have a wattage rating. The higher this rating the more heat they can dissipate.  The wattage in a resistor is the maximum amount of heat energy it can safely dissipate without damage. 
· Demonstration: use a 150 ohm ½ watt and ¼ watt resistors with various currents from a robust power supply.  Note how the heat from the resistors varies.  Verify the value of the resistor as the voltage increases.
· Resistors carrying large currents must be physically large so the heat can radiate quickly to the surrounding air.

· Demonstration: use a small (1/4 watt) and a larger (10 watt) resistor, both having a resistance of 10 ohms. The current through each resistor will be the same.  Note the variation in heat given off by the two resistors.

· Complete Resistors Lab 2 – Resistors and Current
· Suggested homework, Student Activity Sheets 3-2, 3-3.

Electronics Technology and Robotics I Week 8

Resistor Lab 1 – Resistor Color Code

· Purpose:  The purpose of this lab is to acquaint the student with the resistor color code and tolerance calculations. 

· Apparatus and Materials:
· 10 – Fixed Resistors Labeled 1 – 10

· Procedure:
· Using the resistor color code, determine the value of 10 resistors and record the values in the table below.

· Calculate the limits that are within tolerance

· Measure the resistance of each resistor and compare with the coded value.
· Results:

[image: image7.jpg]Resistor

Coded Value

Product of
Tolerance x Value

Upper
Limit

Lower
Limit

Measured Value

olo|x|Nfo|o|afwn| =





· Conclusions:

· Are the measured values of each resistor within the tolerance limits?
Electronics Technology and Robotics I Week 8

Resistor Lab 2 – Resistors and Current
· Purpose:  The purpose of this lab is to demonstrate to the student that resistors restrict current and that an increase in resistance increases the voltage drop across the resistor. 

· Apparatus and Materials:
· 1 – Breadboard with 9 V Supply 

· 3 – Digital Multimeters

· 1 – 1, 10, 22, 47, 68, and 100 Ohm Resistor

· 1 – 7.5 V Lamp with Lamp Base

· 1 – SPDT Switch

· Procedure:
· Assemble the circuit below on the breadboard using 1, 10, 22, 47, 68, and 100 ohm resistors as R1.

· Measure the current for each change in resistor value.
· Calculate the voltage drop across R1 using Ohm’s law.

· Now measure the voltage drop across R1 and the lamp.

· Compare the calculated voltage drop across R1 with the measured value.

· Add the measured voltage drops across R1 and the lamp to get the total measured voltage drop.

· Compare the total measured voltage drop with the measured source.
· Note the brightness variation in each case.

                                                       [image: image8.jpg]R1
Varies

LMP1




                                                       

· Results:

[image: image9.jpg]Voltage Drops Lab 2

Circuit| R1 | Current| Calculate | Measured Measured Total
Voltage Voltage Voltage Measured
Drop R1 Drop R1 Drop LMP1 | Voltage Drop

(OB (WN]|—=





· Conclusions:
· Is the calculated voltage drop across R1 close to the measured voltage drop across R1?

· As the value of R1 is increased, what happens to the voltage drop across R1?

· As the value of R1 is increased, what happens to the brightness of the lamp?  Why?
PAGE  
1

